# Task 06

## Implement the given scenario in Cisco Packet Tracer.

An organization has:

192.168.50.0/24

It needs:

| Department | Hosts Required |
| --- | --- |
| Data Science | 40 |
| Administration | 18 |
| Faculty | 28 |
| Management | 13 |

Determine a suitable prefix and subnet mask for each. Then assign address ranges.

| Department | Prefix | Mask | Network | Host Range | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Data Science | /26 | 255.255.255.192 | 192.168.50.0 | 192.168.50.1-192.168.50.62 | 192.168.50.63 |
| Administration | /27 | 255.255.255.224 | 192.168.50.64 | 192.168.50.65-192.168.50.94 | 192.168.50.95 |
| Faculty | /27 | 255.255.255.224 | 192.168.50.96 | 192.168.50.97-192.168.50.126 | 192.168.50.127 |
| Management | /28 | 255.255.255.240 | 192.168.50.128 | 192.168.50.129-192.168.50.142 | 192.168.50.143 |

## Implement the given scenario in Cisco Packet Tracer.

You are given:

192.168.10.0/24

The organization requires:

- Network A → 100 hosts

- Network B → 50 hosts

- Network C → 20 hosts

- Network D → 10 hosts

Design an appropriate addressing plan. Students must determine:

- Prefix

- Subnet mask

- Network address

- First usable host

- Last usable host

- Broadcast address